Window 10 IOT Get Started

In this tutorial, you can get stared with the window 10 IOT through the Basic Demo, blinking the onboard LED.

Hardware List:

- Raspberry Pi 2

- Arduino Expansion Shield for Raspberry Pi B+

- Magnet Micro USB Cable

- CAT 5 Ethernet Cable

- HDMI Cable

- Micro SD Memory Card 16GB Class10

- USB Keyboard

- USB Mouse

Hardware Connection:

- Plug the “Arduino Expansion Shield for Raspberry Pi B+” onto the Raspberry Pi 2

- Connect Ethernet, mouse, keyboard, HDMI.

- Connect Two boards via USB.

- Insert the Micro SD card into Raspberry Pi.

- Connect USB Power Supply.

- After windows 10 IOT is successfully booted. Some basic device information is there and you can use your mouse to control it.

Set up your PC

- Make sure you are running Windows 10 (version 10.0.10240) or better.

- Download Visual Studio 2015 Here:

https://www.visualstudio.com/downloads/download-visual-studio-vs

- Install It, remember to select the “Universal Windows App Development Tools”:

- Enable developer mode on your windows 10

Blink Demo:

- Download the “DFrobotWindowIOTTempelate”

https://github.com/DFRobot/DFrobotWindowIOTTempelate

- Open DFrobotWindowIOTTempelate Project in the DFrobotWindowIOTTempelate folder.

- Change the Target to ARM:

- Change the target device to Remote Machine:

- Select your raspberry Pi on the network

- Main Code is in “MainPage.xaml.cs”.

using Windows.UI.Xaml;

using Windows.UI.Xaml.Controls;

using Microsoft.Maker.RemoteWiring;

using Microsoft.Maker.Serial;

using System;

using System.Diagnostics;

namespace DFrobotWindowIOTTempelate

{

public sealed partial class MainPage : Page

{

UsbSerial usb;                          //Handle the USB connction

RemoteDevice arduino;                   //Handle the arduino

private DispatcherTimer blinkTimer;     //Timer for the LED to blink every one second

private const int LED_PIN = 13;         //Pin number of the on board LED

private PinState ledState;              //Pin state of the LED

public MainPage()

{

this.InitializeComponent();

//USB VID and PID of the "Arduino Expansion Shield for Raspberry Pi B+"

usb = new UsbSerial("VID_2341", "PID_8036");

//Arduino RemoteDevice Constractor via USB.

arduino = new RemoteDevice(usb);

//Add DeviceReady callback when connecting successfully

arduino.DeviceReady += onDeviceReady;

//Baudrate on 57600 and SerialConfig.8N1 is the default config for Arduino devices over USB

usb.begin(57600, SerialConfig.SERIAL_8N1);

}

private void onDeviceReady()

{

//After device is ready this function will be evoked.

//Debug message "Device Ready" will be shown in the "Output" dialog.

Debug.WriteLine("Device Ready");

var action = Dispatcher.RunAsync(Windows.UI.Core.CoreDispatcherPriority.Normal, new Windows.UI.Core.DispatchedHandler(() =>

{

setup();

}));

}

private void setup()

{

//Set the initial state of the led.

ledState = PinState.LOW;

//Set the pin mode of the led.

arduino.pinMode(LED_PIN, PinMode.OUTPUT);

//Set the timer to schedule blink() every one second.

blinkTimer = new DispatcherTimer();

blinkTimer.Interval = TimeSpan.FromMilliseconds(1000);

blinkTimer.Tick += blink;

blinkTimer.Start();

}

private void blink(object sender, object e)

{

if (ledState == PinState.HIGH)  //LED state is HIGH.

{

//Turn off the LED.

arduino.digitalWrite(LED_PIN, PinState.LOW);

//Show the message in the Output dialog.

Debug.WriteLine("OFF");

//Set local LED state to Low.

ledState = PinState.LOW;

}

else    //LED state is LOW.

{

//Turn on the LED.

arduino.digitalWrite(LED_PIN, PinState.HIGH);

//Show the message in the Output dialog.

Debug.WriteLine("ON");

//Set local LED state to Low.

ledState = PinState.HIGH;

}

}

}

}

- The onboard LED start to blink. Congratulations!!